Cicci Research

Arkeo All-in-One API

v0.1.0

11/11/2025

Contents

Introduction	2

List of commands	3

MAIN	4

Command Summary	4

Commands Details	4

ROUTINE	6

Command Summary	6

Commands Details	6

Routine Specific Commands	10

TMPCTL	11

MUX	12

Command Summary	12

Commands Details	12

Examples	15

Python	15

Changelog	16

# Introduction

This API facilitates remote control of the Arkeo All-in-One software. Communication happens using the TCP protocol on port 6360, enabling users to perform a variety of commands manage measurements. Messages are formatted as JSON, its structure depends on the specific commands.

Each command must sent as a JSON string in the form
{
  "target":<TARGET>,
  "command":"<COMMAND>",
  "parameter":"<PARAMETERS>",
  "request_id":42          // optional
}

Each command must be preceded by a 4-byte integer indicating the total length of the command. After each command, a reply is always sent back with the following structure:

{

"status": "OK" | "ERROR",

"data": { ... },         // present if status = "OK"

"error": { ... },        // present if status = "ERROR"

"request_id": <same as request>

}

The "data" format depends on the command. If an unknown command is sent or the JSON is malformed, "status" will indicate "error" instead and the error key is send instead of the data key.

# List of commands

Each "target" has its own set of commands. The routine target may have custom commands based on which routine is active. Sending a command not in the list of available commands for the selected target, returns “invalid command”.

The following sections details every available command, separated by targets. The following targets are available

| MAIN | Commands related to the main launcher |
| --- | --- |
| ROUTINE | Commands for the active routine |
| TMPCTL | Commands for a temperature controller |
| MUX | Commands for a multiplexer |

Each command detail includes an example request and response. "target", "status" and "request_id" are not included in the examples, but are implied.

## MAIN

These commands can always be called.

### Command Summary

| Command | Description |
| --- | --- |
| StartRoutine | Starts the selected routine |
| GetActiveRoutine | Returns the active routine. Only one routine may be active at a time |
| GetAvailableRoutines | Returns an list of all available routines |

### Commands Details

#### Command: StartRoutine

Description: Launches a routine.

Parameters

| Key | Type | Required | Description |
| --- | --- | --- | --- |
| routine | string | ✓ | Name of the routine to load. |

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string | OK if routine started |

Example Request

{

"command": "StartRoutine",

"params": { "routine": "JV" },

}

Example Response

{ "state":"OK" }

Command: GetActiveRoutine

Description: Returns the active routine. Only one routine may be active at a time.

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string | Name of the active routine, or empty if none. |

Example Request

{ "command": "GetActiveRoutine" }

Example Response

{"routine_name": "JV" }

#### Command: GetAvailableRoutines

Description: Returns an array of all available routines.

Response

| Key | Type | Description |
| --- | --- | --- |
| routines | string[] | Array of available routine names. |

Example Request

{ "command": "GetAvailableRoutines" }

Example Response

{ "routines":["JV","MPPT"] }

## ROUTINE

The following commands can only be called if a routine is active. If no routine is active, any of these commands return error 4006: “no active routine” as an error object.

### Command Summary

| Command | Description |
| --- | --- |
| StartMeasurement | Starts the measurements |
| StopMeasurement | Stops the ongoing measurement |
| CloseRoutine | Stops any ongoing measurement and close the routine |
| GetTestData | Returns the measurement data |
| GetTestStatus | Returns the current status of the routine |
| ClearErrors | Clears any errors when the routine is in the error state |
| GetSettings | Returns the routine configuration |
| ApplySettings | Applies a configuration |
| SetInfo | Applies user and device name |
| GetCustomCommands | Returns a list of custom commands available for the active routine |

### Commands Details

#### Command: StartMeasurement

Description: Starts the measurements. This command returns an error when the routine is not in the Ready state.

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string |  |

Example Request

{"command": "StartMeasurement" }

Example Response

{ "state":"OK" }

#### Command: StopMeasurement

Description: Stops the measurements. This command returns an error when the routine is not in the Starting or Running state.

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string |  |

Example Request

{"command": "StopMeasurement" }

Example Response

{ "state":"OK" }

#### Command: CloseRoutine

Description: Stops any ongoing measurement, clears all errors and close the routine.

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string |  |

Example Request

{"command": "CloseRoutine" }

Example Response

{ "state":"OK" }

#### Command: GetTestData

Description: Returns the measurement data. The format depends on the active routine. For more details see

#### Command: GetTestStatus

Description: Returns the current status of the routine.

Response

| Key | Type | Description |
| --- | --- | --- |
| routine_status | string | Current state of the routine. One of: Initializing, Ready, Starting, Running, Error. |

Example Request

{"command": "GetTestStatus" }

Example Response

{ "routine_status":"Running" }
{ "routine_status":"Error", "error":{"code":1000, "message": "Hardware error"}}

#### Command: ClearErrors

Description: Clears any errors when the routine is in error. After this command, the routine returns to the Ready state.

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string |  |

Example Request

{"command": "GetTestStatus" }

Example Response

{ "state":"OK" }

#### Command: GetSettings

Description: Returns the routine configuration. The format depends on the active routine.

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string |  |

Example Request

{"command": "GetSettings" }

Example Response

{

"device_type":"NI-SMU",

"device_settings":{"Compliance (A)":0.01},

"scan_settings":{

"scan":{

"Start (V)":-0.2,

"End (V)":1,

"Step (V)":0.02,
      ...
   }
}

#### Command: ApplySettings

Description: Applies a configuration. The configuration JSON must match the active routine. It is recommended to use GetSettings to obtain the correct JSON.

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string |  |

Example Request

{

"device_type":"NI-SMU",

"device_settings":{"Compliance (A)":0.01},

"scan_settings":{

"scan":{

"Start (V)":-0.2,

"End (V)":1,

"Step (V)":0.02,

...

}

}
Example Response

{ "state":"OK" }

#### Command: SetInfo

Description: Set the user name, device name and device area.

Parameters

| Key | Type | Required | Description |
| --- | --- | --- | --- |
| user_name | string | ✓ | User name |
| device_name | string | ✓ | Device name |
| device_area | float |  | Device Area in cm² |

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string | OK |

Example Request

{

"command": "SetInfo",

"params": { "user_name": "User",

"device_name": "Sample",

"device_area": 1.23 },

}

Example Response

{ "state":"OK" }

#### Command: GetCustomCommands

Description: Returns a list of custom commands available for the active routine.

Response

| Key | Type | Description |
| --- | --- | --- |
| CustomCommands | string[] | List of custom commands |

Example Request

{"command": "GetCustomCommands"}

Example Response

{"CustomCommands":["AcquireDark","AcquireReference","AcquireSingle","SetLaser","AutoExposure"]}

### Routine Specific Commands

Some routines have specific commands. They can only be called when that routine is active. Otherwise an “invalid command” is returned.

The following sections detail the specific commands for each routine.

#### JV

##### Settings JSON

{

"device_type":"NI-SMU",

"device_settings":{"Compliance (A)":0.01},

"scan_settings":{

"scan":{

"Start (V)":-0.2,

"End (V)":1.2,

"Step (V)":0.02,

"Rate (V/s)":0.2,

"Order":"Forward then Reverse",

"Auto Voc":true,

"Exceed Voc (%)":0

},

"precondition":{

"Condition":"None",

"Value":1,

"Hold Time (s)":1,

"Hold Voltage (V)":-0.2

},

"device":{"Area (cm2)":1,"Inverted":false},

"light":{

"Source":"None",

"Irradiance (mW/cm2)":100,

"Level (%)":100

},

"light_config":{"LED Level":1}

},

"sweep_settings":[]

}

##### Data JSON

{

"user":"User",

"device":"Sample",

"temperature":0,

"test":"",

"time":"1904-01-01T00:00:00.000Z",

"sweep":[],

"data_schema":[

{"name":"Voltage","unit":"V"},

{"name":"Current","unit":"mA/cm2"}

],

"parameter_schema":[

{"name":"Voc","unit":"V"},

{"name":"Jsc","unit":"mA/cm2"},

{"name":"V_MPP","unit":"V"},

{"name":"J_MPP","unit":"mA/cm2"},

{"name":"P_MPP","unit":"mW/cm2"},

{"name":"Rseries","unit":"Ohm"},

{"name":"Rshunt","unit":"Ohm"},

{"name":"Fill Factor","unit":"%"},

{"name":"Efficiency","unit":"%"}

],

"scans":[

{

"name":"forward",

"file":"D:\\Arkeo\\Results\\Cicci Research\\2025_10_30\\Sample\\JV\\2025-10-30_14.51.57_JV_Sample.txt",

"data":[

[-0.100211457860631,1.14872582495027E-5],

[-0.0800413256365645,1.19740465092449E-5],

[-0.0600455286506167,1.18040040349514E-5],...

],

"parameters":[

0.199957825294649,

0.0112409373200826,

0.128680169428345,

0.00767840281811049,

9.88058175573541E-4,

7141.88672920349,

89045.9727026382,

43.9583704939779,

9.88058175573541E-4

],

"sweep_indices":[-1]

},

{

"name":"reverse",

"file":"D:\\Arkeo\\Results\\Cicci Research\\2025_10_30\\Sample\\JV\\2025-10-30_14.51.57_JV_Sample.txt",

"data":[

[0.199999938425143,7.00119324713674E-7],

[0.18004202880006,3.61618105053908E-6],

[0.160041917080911,5.6967007360125E-6],...

],

"parameters":[

0.199999938425143,

0.0115814924533883,

0.129775601459748,

0.00794798653972136,

0.00103145473358632,

6632.67377340069,

94258.7456476277,

44.5303165933094,

0.00103145473358632

],

"sweep_indices":[-1]

}

]

}

##### Specific Commands

None

#### IPCE

##### Settings JSON

##### Data JSON

##### Specific Commands

Command: SetShutter

Description: Open or close the shutter

Parameters

| Key | Type | Required | Description |
| --- | --- | --- | --- |
| Shutter Status | bool | ✓ | True: Shutter open False: Shutter closed |

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string | OK |

Example Request

{

"command": "SetShutter",

"params": { "shutter_status": true}

}

Example Response

{ "state":"OK" }

#### IPCE

##### Settings JSON

##### Data JSON

##### Specific Commands

## TMPCTL

These commands can only be called when a temperature controller is available.

TBA

## MUX

These commands can only be called when a device multiplexer is available

### Command Summary

| Command | Description |
| --- | --- |
| Connect | Connects the multiplexer |
| SetState | Set the state of each multiplexer channel |
| GetStatus | Gets the status of the multiplexer |
| Disconnect | Disconnects the multiplexer |

### Commands Details

#### Command: Connect

Description: Connects the multiplexer

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string | Connection status |

Example Request

{"command": "Connect"}

Example Response

{ "state":"OK" }
{ "state":"error", "error":{"code":1002, "message": "Multiplexer not found"}}

#### Command: SetState

Description: Set the state of each multiplexer channel.

Parameters

| Key | Type | Required | Description |
| --- | --- | --- | --- |
| State | bool[] | ✓ | Boolean array of state for each channel |

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string | OK |

Example Request

{

"command": "SetState",

"params": { "state": [false, true, false]}

}

Example Response

{ "state":"OK" }

{ "state":"error", "error":{"code":1004, "message": "Unable to set channels"}}
Command: GetStatus

Description: Gets the status of the multiplexer.

Response

| Key | Type | Description |
| --- | --- | --- |
| connected | Boolean | Whether the multiplexer is connected |
| firmware | String | Firmware of the connected multiplexer |
| nrRelays | Int | Number of available channels |
| state | Bool[] | Boolean array of state for each channel |

Example Request

{ "command": "GetStatus" }

Example Response

{

"connected":true,

"firmware":"Multiplexer_v1.1",

"nrRelays":4,

"status":[false, false, false, true]

}

#### Command: Disconnect

Description: Disconnects the multiplexer.

Response

| Key | Type | Description |
| --- | --- | --- |
| state | string | OK |

Example Request

{ "command": "Disconnect" }

Example Response

{ "state":"OK" }

# Examples

## Python

import socket

import json

def exampleCommand(command, data=''):

with socket.socket(socket.AF_INET, socket.SOCK_STREAM) as sock:

sock.connect(('localhost', 6340))

message = json.dumps({'command': command, 'data': data})

# Prepare the length of the string

length = len(message)

length_bytes = length.to_bytes(4, byteorder='big')  # 4 bytes to represent the length

sock.sendall(length_bytes) # Send the length of the string

sock.sendall(message.encode('utf-8')) # Send the string

# Receive response

response = sock.recv(1024)

print('Received:', response.decode('utf-8'))

exampleCommand('GetAvailableRoutines')

# Changelog

v0.1
Initial version